Fire Pump Operation

These are the basics for setting up, starting and running the fire pump

SETUP

- Place the pump on a solid surface, close enough for the SUCTION HOSE to reach the water.

- Check there is gas in the tank.

- Prime pump - add water into prime port until it exits inlet port (use jug or boat bailer)

- Attach the SUCTION HOSE to the pump INLET and lock in place.

SUCTION HOSE w STRAINER

+

INLET

- Completely submerge the STRAINER in the water – may need weighing down.

- Unroll the DISCHARGE HOSES, make sure there are no twists or kinks.

- Attach the female hose end to the pump OUTLET, lock in place.

  - Multiple discharge hoses can be coupled together.

- Attach the NOZZLE to the end of the discharge hose(s) and lock in place.

- Prime pump via prime port – use lake water/boat bailer/cup etc – replace prime cap

PRIME PORT

NOZZLE

Discharge Hoses,

(Blue or White)

+

+

OUTLET

OR

You are ready to start the engine! Have someone holding the nozzle!

STARTING THE PUMP (TGIC)

Note: pumps in tubular metal cages do NOT have a gas on/off or an ignition switch

Throttle    /    Gas    /    Ignition    /    Choke

Throttle to the left

Gas to the right

Ignition to the “ON” position

THROTTLE

CHOKE

IGNITION

GAS

Choke to the left

- Pull the starter cord until pump starts.

- Move the choke to the right.

- The pump will self-prime up the suction hose if you added water to the pump via priming port, it takes a minute!

- Have someone hold the nozzle, it’s powerful once the water flows!

- Aim water at the base of the fire, adjust nozzle spray pattern as needed